GUIDE FOR WATER SAMPLING SKILL FROM A TAP FOR BACTERIOLOGICAL ANALYSIS

AIM: Gaining sampling skill from a tap for bacteriological analysis

MATERIALS: Sterilized bottle, cigarette lighter

| 1 | Wash the hands |
| --- | --- |
| 2 | Sterilize the tap for 1 minute with cigarette lighter |
| 3 | Open the tap before sampling. Carefully turn on the tap and allow water to flow at medium rate for 3 - 5 minutes |
| 4 | Carefully remove the cap and protective cover from the bottle, taking care to prevent entry of dust that may contaminate the sample |
| 5 | Hold the bottle immediately under the water jet to fill it. |
| 6 | A small air space should be left to allow mixing before analysis. |
| 7 | Replace the bottle cap. |
| 8 | Fill the label on the bottle* |
| 9 | Take the water to the laboratory for the bacteriological analysis** |

*The place where and when the sample is taken should be written on the label.

** The sample should be reached to the laboratory within maximum 6 hours at room temperature or 48 hours in ice.

GUIDE FOR MEASURING RESIDUAL CHLORINE IN WATER

AIM: Gaining the skill of measuring residual chlorine in water

MATERIALS: Comparator, orthotolidine solution, sample of water.

| 1 | If the sample will be taken from a tap, allow water to flow at highest rate for 2 - 4 minutes. |
| --- | --- |
| 2 | Check the cleanness of the comparator tubes. |
| 3 | Fill the “Confirmation tube” with the chlorinated water sample up to the 10-ml mark. |
| 4 | Add 0.5 ml orthotolidine solution into the tube with a dropper. |
| 5 | Close the brim of the tube with your finger and shake it for 5-6 times. |
| 6 | Place the tube to the socket at the upper right part of comparator. |
| 7 | Fill the second tube ( tube)also with the chlorinated water sample up to the 10-ml mark |
| 8 | Place the second tube “Equilibration tube” to the socket at the upper left part of comparator. |
| 9 | Color comparison is done through direct day light from North. |
| 10 | Rotate the color disc. |
| 11 | Read the result from the hole placed at the upper part of the comparator when the color of the sample is paired off with the color standard of the disk. |

REFERENCES:

- OĞUR R, TEKBAŞ ÖF, HASDE M. KLORLAMA REHBERİ (İÇME VE KULLANMA SULARININ KLORLANMASI). Ankara, 2005. ISBN: 1302-0471

- http://ww.ahsl.gov.tr/index.php/numune-alma-reber.html

- http://www.hssgm.gov.tr/menu/mevzuat/yonerge_genelge.aspx