GUIDE FOR WATER SAMPLING SKILL FROM A TAP FOR BACTERIOLOGICAL ANALYSIS

AIM: Gaining sampling skill from a tap for bacteriological analysis

MATERIALS: Sterilized bottle, lighter, pen, soap

GENERAL INFORMATION: Aims of bacteriological water analysis are;

1. To determine and control the contamination of water by microorganisms

2. To determine fecal contamination by means of organisms showing fecal contamination and search for origin

3. To prevent and control water originated epidemics

4. To provide bacteriological safe water consumption

The sampling period and the number of samples may be determined by considering the total population.

| Population |  | Maximum period between samples (Day) | Minimum sample number |
| --- | --- | --- | --- |
| <20,000 |  | 30 | 1 sample for each 5,000 |
| 20,000-50,000 |  | 15 | 1 sample for each 5,000 |
| 50,000-100,000 |  | 4 | 1 sample for each 5,000 |
| >100,000 |  | 1 | 1 sample for each 10,000 |
| Bottles supplied by the laboratory should be used for water analysis and a sample of 500 mL should be taken. The sample should be put into the bottles containing 118 mg/L sodium thiosulfate if the water is chlorinated. |  |  |  |
| 1 | Wash the hands with soap |  |  |  |
| 2 | Sterilize the tap for 1 minute with the flame of a lighter or a cotton wrapped stick dipped into alcohol |  |  |  |
| 3 | Open the tap before sampling. Carefully turn on the tap and allow water to flow at medium rate for 3 - 5 minutes |  |  |  |
| 4 | Carefully remove the cap and protective cover from the bottle, taking care to prevent entry of dust that may contaminate the sample |  |  |  |
| 5 | Hold the bottle immediately under the water jet to fill it up to 500 mL level |  |  |  |
| 6 | A small air space (1/10 of bottle) should be left to allow mixing before analysis. |  |  |  |
| 7 | Replace the bottle cap. |  |  |  |
| 8 | Fill the label on the bottle. The place where and when the sample is taken should be written on the label. |  |  |  |
| 9 | Take the water to the laboratory for the bacteriological analysis (The sample should be reached to the laboratory within maximum 12 hours at 5 (±3)° C in cold chain |  |  |  |

GUIDE FOR MEASURING RESIDUAL CHLORINE IN WATER

AIM: Gaining the skill of measuring residual chlorine in water

MATERIALS: Comparator, ortho-toluidine solution, sample of water

GENERAL INFORMATION: Municipalities  disinfect the city water and Control Center measures the residual chlorine level twice in a day. Residual chlorine level should be  measured at the storage leaving point if the water is stored before the distribution. The residual chlorine level should be  between 0.2 - 0.5 ppm  at the end point, if it is not the municipality should be warned.

CONFIRMATION TUBE: The tube which the chlorinated water sample and 0.5 mL of ortho-toluidine solution are put into.

EQUILIBRATION TUBE: The tube which is filled with the chlorinated water sample and used to compare the color change occurred in the confirmation tube related to ortho-toluidine solution.

| 1 | If the sample will be taken from a tap, allow water to flow at highest rate for 2 - 4 minutes. |
| --- | --- |
| 2 | Check the cleanness of the comparator tubes. |
| 3 | Fill the “Confirmation tube” with the chlorinated water sample up to the 10 mL mark. |
| 4 | Add 0.5 mL ortho-toluidine solution into the tube with a dropper. |
| 5 | Close the brim of the tube with your finger and shake it for 5-6 times. |
| 6 | Place the tube to the socket at the upper right part of comparator. |
| 7 | Fill the second tube "Equilibration tube" also with the chlorinated water sample up to the 10-ml mark |
| 8 | Place the second tube “Equilibration tube” to the socket at the upper left part of comparator. |
| 9 | Color comparison is done through direct day light from North. |
| 10 | Rotate the color disc. |
| 11 | Read the result from the hole placed at the upper part of the comparator when the color of the sample is paired off with the color standard of the disk. |

REFERENCES:

- OĞUR R, TEKBAŞ ÖF, HASDE M. KLORLAMA REHBERİ (İÇME VE KULLANMA SULARININ KLORLANMASI). Ankara, 2005. ISBN: 1302-0471

- http://ww.ahsl.gov.tr/index.php/numune-alma-reber.html

- http://www.hssgm.gov.tr/menu/mevzuat/yonerge_genelge.aspx